| Table 1. Methods for diagnosing Demodex spp. and assessment of the suitability of their application in a specialized outpatient parasitological laboratory. |  |  |  |
| --- | --- | --- | --- |
| Methods (source) | Advantages | Disadvantages | Assessment |
| I. Methods for skin |  |  |  |
| Superficial scotch-tape impression/biopsy (1,2,7,8,9) | - Not invasive - Suitable for all regions of the face - Safe and easy (can be done at home) - Does not require immediate microscopy (allows transport) - Can be performed in outpatient conditions (at home) - Fast and cheap | - Only reaches the superficial layer (less informative for active pustules and D. brevis) | Suitable |
| Direct microscopy after skin scraping (1,7,8) | - Reaches deeper into the skin - Useful for pustules - Suitable for all regions of the face - Standardized for 1 cm2 | - Only performed by a dermatologist - Requires immediate microscopy (does not allow transport) | Not suitable |
| Superficial skin biopsy with cyanoacrylate adhesive (1,7,8,9,11) | - Reaches deeper skin layer - Standardized for 1 cm2 | - Inflicts epithelial trauma - Depth is achieved with repeated application - Difficult to perform in some regions of the face (nose) - Allergic reactions to the adhesive are possible - Requires microscopy immediately (does not allow transport) | Not suitable |
| Skin biopsy with histological preparation (2,7,8,9) | - The most informative - Assesses penetration into the dermis | - Not suitable for examining large skin areas or multiple areas - Performed by a trained specialist - Requires local anaesthesia - Causes trauma and can leave scars - Requires histological staining - Time-consuming | Not suitable |
| Dermatoscopy (7,13,15) | - Not invasive - Suitable for all regions of the face - Can be standardized for 1 cm2 | - Examines only the surface (less informative for active pustules and for D. brevis) - Performed by a dermatologist - Requires specialized equipment | Not suitable |
| In vivo confocal laser or refractive microscopy (2,7,9,11,14,19) | - Not invasive - Reach deeper skin layers - Can be standardized for 1 cm2 | - Performed only by a qualified specialist - Requires highly specialized and expensive equipment | Not suitable |
| - II. Methods for eyelids and eyelashes |  |  |  |
| Native microscopy after epilation of eyelashes (1,5,6,12,16,17,19) | - Damaged eyelashes with circular dandruff can be selected - Mites remain alive - Standardized - Can be performed in outpatient conditions (at home) - Fast and cheap | - Invasive - Sensitivity upon extraction - Difficult for transportation (in its original form) | Suitable |
| Epilation of eyelashes with the addition of solutions (1,5,6,12,19) | - Damaged eyelashes with circular dandruff can be selected - Break down detritus (KOH, 100% alcohol) and/or make mites easier to distinguish (methylene blue, fluorescein) - Standardized | - Invasive - Require microscopy immediately (does not allow transportation) - The sample can be dispersed and/or lost - Can change the morphology of the mites | Not suitable |
| Confocal scanning laser microscopy (5,6,7,17,12,18) | - Not invasive - High informativeness deeper in the follicles - Standardized | - Performed by a qualified specialist - Difficult for examination of the eyelids - Requires highly specialized and expensive equipment | Not suitable |