| Method | Type of system | Sink conditiones | Time | Release media | Volume, mL | Rotation speed, rpm | Temperature, oC | Flow-in speed | Flow-out speed | Sampling and mesurement |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| I. EP Apparatus 2: Paddle method (Method I) | closed | no | 0-1h | HAS pH 1,5 | 900 | 100 | 37±0.5 | x | x | Samples from the release media were withdrawn at chosen times (through 5 µm pore size filter) and analyzed spectrophotometrically for diosmin and hesperidin content. Concentration [mg/L] was extrapolated to [mg] drug released in 900 mL and % drug released was presented knowing the labeled drug content in tablets. |
|  |  |  | 1-4h | PBS pH 4,5 | 900 |  |  |  |  |  |
|  |  |  | 4-24h | PBS pH 7,2 | 900 |  |  |  |  |  |
| II. Flow-through cell resambling setting (Method II) | opened | yes | 0-1h | HAS pH 1,5 | 50 | x | approximately 37 | 4 mL/min (80 drops per min) | 4 mL/min (80 drops per min) | The buffer solution, collected in the receiving compartiment, was analyzed for diosmin and hesperidin content at chosen times. Drug concentration [mg/L] was extrapolated to % drug released, knowing the exact volume of the collected buffer at the time of the measurement and the labeled drug content. |
|  |  |  | 1-4h | PBS pH 4,5 | 50 |  |  |  |  |  |
|  |  |  | 4-12h | PBS pH 7,2 | 50 |  |  |  |  |  |

Table 2: In vitro drug release tests- parameters of the studies