Table 3: Quality of tablets

| PRODUCT | UNIFORMITY OF MASS |  |  | UNIFORMITY OF DOSAGE UNITS |  |  |  | DISINTEGRATION |  | RESISTANCE TO CRUSHING |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Average mass, mg | ±SDa, % | EP Acceptance criteria, ±SD% | Average drug content, %b | ±SD, % | Acceptance value (AV)c | EP Acceptance criteria, AV | Time, min | EP Acceptance criteria, min | Hardness, N | ±SD, N |
| Product Am | 688 | 0.65 | ≤5.00 | 108.32 | 3.65 | 1.94 | ≤15 | 17.56 | ≤30 | 195 | 19 |
| Product B | 848 | 0.53 |  | 103.20 | 7.76 | 16.93 |  | 73.00 |  | >300d | d |
| Product C | 634 | 2.39 |  | 99.05 | 12.16 | 31.64 |  | 19.56 |  | 218 | 13 |
| Product D | 976 | 0.56 |  | 109.01 | 4.90 | 4.24 |  | 3.21 |  | >300d | d |
| Product Em | 944 | 0.58 |  | 124.22 | 8.29 | -2.83 |  | 10.20 |  | 256 | 16 |
| Product Fm | 730 | 1.37 |  | 109.99 | 8.88 | 12.82 |  | 3.30 |  | 189 | 24 |
| Product G | 1278 | 0.65 |  | 109.15 | 5.79 | 6.25 |  | 3.22 |  | >300d | d |

a SD - standard deviation

b Drug content found in tablets, expressed as percent (%) of the labeled content

c AV was calculated according to the EP recommendations – chapter 2.9.40 Uniformity of dosage units, Mass variation

d The value of hardness [N] has excelled the apparatus limitations of detection (10-300 N), SD could not be calculated